exacqVision® Surveillance Keyboard

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

# DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

# LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

# LEVEL 2__28 23 00 VIDEO SURVEILLANCE

# LEVEL 3__28 23 13 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

# PART 2 – PRODUCTS

# 2.01	SURVEILLANCE KEYBOARD DESCRIPTION

- The surveillance keyboard shall be an add-on to a video management system (VMS).

- The surveillance keyboard shall plug into an available USB port.

- The surveillance keyboard shall operate on Windows, Linux and Macintosh systems.

- The surveillance keyboard shall be part of an integrated system that includes custom firmware to integrate with VMS software.

- The surveillance keyboard shall include a three-axis hall-effect joystick, a 10-button numeric keypad, 16 user-defined buttons, jog/shuttle dial and USB 1.1 interface. The jog/shuttle shall be able to travel 360 degrees. The surveillance keyboard shall have LED-illuminated buttons.

2.02	SPECIFICATIONS AND MODEL NUMBER

A) All surveillance keyboards shall have the following specifications:

1. Dimensions:			9 x 11x 4.36 inches (228.35 x 279.40 x 110.74 mm)
		 2. Weight:			1.33 kg (47 oz)
		 3. Operating Temperature:	-25° C to 85° C
		 4. Power Consumption:		Via USB interface (5v DC), 1A max
		 5. Regulatory:			CE, FC, RoHS
		 6. Interface:			USB, 2 m cable length
		 7. Keyboard Housing:		High impact ABS
		 8. Joystick Performance:	Hall-effect three-axes joystick
                                                                              X/Y/Z for positioning control
                                  a. Travel:			X/Y axis +/- 18°
                                                                              Z axis +/- 40°
                                  b.  Centering:		single sprint, omni-directional
                                  c.  Shaft:			Stainless steel
                                  d.  Boot:			Neoprene
                                  e.  Handle:			Glass-filled nylon
		 9. Jog/Shuttle Performance:	Spring-loaded shuttle ring travel +/- 40°
                                                                              Smooth action knob rotates 360°
		10. Pushbutton Performance:	27 pushbuttons: 11 fixed and 16 user-programmable
                                 a.  Lighting:			High efficiency LED
		       b.  Material:			Silicon

B) Model Number:

1. 5000-50100

C) Warranty:

1. One year on parts and labor